Packet Routing System

Project Documentation

Names:

Youssef Hossam EldeenMohamed Elzainy

Fares Nagah Ibrahim

Yehia Omar

Course: Digital IC Design Using FPGA

# 1. Introduction

In most digital systems, more than one part of the chip needs to send data to shared resources at the same time. A single bus cannot serve four sources at once without collisions, so some form of arbitration and routing is needed between the sources and the destinations. This project implements a small router that solves exactly that problem for a system with four input ports and four output ports.

The design accepts data from four independent write ports, each backed by its own synchronous FIFO buffer. A round-robin arbiter monitors the four FIFOs and grants access to the shared output path in a fair, rotating order, so no single port can starve the others. Once a packet is granted access, it passes through a packet routing block that reads the destination bits embedded in the packet and forwards it to the correct one of four output ports. Three of the output ports are connected to simple RAM-based memory nodes that model slave devices on the network, while the fourth port is exposed directly to the testbench so that responses coming back from the memory nodes can be observed and checked.

## 1.1 Objective

The objective of this project is to design and verify a 4-port packet router at the RTL level using Verilog, covering the full data path from FIFO buffering and arbitration through to packet routing and delivery, and to confirm correct operation through simulation and synthesis

## 1.2 Project Overview

The system is built up from a small set of reusable blocks. A generic synchronous FIFO (FIFO_Memory.v) is instantiated four times (Four_FIFO_Buffers.v) to buffer incoming data on each port. A round-robin arbiter (RR_Arbiter.v) decides, cycle by cycle, which non-empty FIFO is allowed to send its data out, and this is combined with the FIFO buffers in FIFO_Arbiter_System.v. The arbitrated data stream is then handed to the Packet_Routing.v module, which decodes the destination field of each packet and steers it to the correct output port. These pieces come together in Router_TopModule.v, which is the top-level router.

Around the router, network.v adds three memory nodes (ram.v) that receive packets, service them, and send response packets back into the network, forming a small closed network rather than just a router in isolation. Port 3 is left open and connects straight to the testbench (n_tb.v), which drives input packets and checks that the corresponding responses come back correctly.

# 2. System Architecture

## 3. Design Files

## 3.1 FIFO_Memory.v — Synchronous FIFO

module sync_fifo #(

parameter WIDTH = 8,

parameter DEPTH = 32

) (

input clk, rst_n,

input wr_en, r_en,

input [WIDTH - 1 : 0] data_in,

output empty, full,

output reg [WIDTH - 1 : 0] data_out

);

reg [$clog2(DEPTH) - 1 : 0] w_ptr, r_ptr; // $clog2 is ceiling of log base 2

reg [$clog2(DEPTH + 1) - 1 : 0] cnt;

reg [WIDTH - 1 : 0] mem [0 : DEPTH - 1];

assign empty = (cnt == 0);

assign full = (cnt == DEPTH);

always @(posedge clk) begin

if(!rst_n) begin

w_ptr <= 0;

r_ptr <= 0;

data_out <= 0;

cnt <= 0;

end else begin

if (wr_en && !full) begin

mem [w_ptr] <= data_in;

w_ptr <= w_ptr + 1;

end

if (r_en && !empty) begin

data_out <= mem[r_ptr];

r_ptr <= r_ptr + 1;

end

case ({wr_en && !full , r_en && !empty})

2'b10: begin

cnt <= cnt + 1;

end

2'b01: begin

cnt <= cnt - 1;

end

default: cnt <= cnt;

endcase

end

end

endmodule

## 3.2 Four_FIFO_Buffers.v — Four FIFO Buffers

module Four_Fifo_Buffers #(

parameter DATA_WIDTH = 40

) (

input clk, rst_n, wr_en0 , wr_en1, wr_en2, wr_en3,

input r_en0, r_en1, r_en2, r_en3,

input [DATA_WIDTH - 1 : 0] data_in0, data_in1, data_in2, data_in3,

output empty0, empty1, empty2, empty3,

output full0, full1, full2, full3,

output [DATA_WIDTH - 1 : 0] data_out0, data_out1, data_out2, data_out3

);

sync_fifo #(

.WIDTH(DATA_WIDTH),

.DEPTH(16)

) FIFO0 (

.clk(clk),

.rst_n(rst_n),

.wr_en(wr_en0),

.r_en(r_en0),

.data_in(data_in0),

.empty(empty0),

.full(full0),

.data_out(data_out0)

);

sync_fifo #(

.WIDTH(DATA_WIDTH),

.DEPTH(16)

) FIFO1 (

.clk(clk),

.rst_n(rst_n),

.wr_en(wr_en1),

.r_en(r_en1),

.data_in(data_in1),

.empty(empty1),

.full(full1),

.data_out(data_out1)

);

sync_fifo #(

.WIDTH(DATA_WIDTH),

.DEPTH(16)

) FIFO2 (

.clk(clk),

.rst_n(rst_n),

.wr_en(wr_en2),

.r_en(r_en2),

.data_in(data_in2),

.empty(empty2),

.full(full2),

.data_out(data_out2)

);

sync_fifo #(

.WIDTH(DATA_WIDTH),

.DEPTH(16)

) FIFO3 (

.clk(clk),

.rst_n(rst_n),

.wr_en(wr_en3),

.r_en(r_en3),

.data_in(data_in3),

.empty(empty3),

.full(full3),

.data_out(data_out3)

);

endmodule

## 3.3 RR_Arbiter.v — Round Robin Arbiter

module RR_Arbiter(

input clk, rst_n,

input empty0, empty1, empty2, empty3,

input Router_Ready,

output grant0, grant1, grant2, grant3

);

reg [3:0] grant_all;

assign {grant3 , grant2 , grant1 , grant0} = grant_all;

reg [1:0] current_state, next_state;

localparam [1:0] S_FIFO0 = 2'b00,

S_FIFO1 = 2'b01,

S_FIFO2 = 2'b10,

S_FIFO3 = 2'b11;

// current state logic

always @(posedge clk) begin

if (!rst_n) begin

current_state <= S_FIFO0;

end else begin

current_state <= next_state;

end

end

// next state logic

always @(*) begin

next_state = current_state;

if (Router_Ready) begin

case (grant_all)

4'b0001: next_state = S_FIFO1;

4'b0010: next_state = S_FIFO2;

4'b0100: next_state = S_FIFO3;

4'b1000: next_state = S_FIFO0;

default: next_state = current_state;

endcase

end

end

// Output logic

always @(*) begin

case (current_state)

S_FIFO0: begin

if (!empty0)

grant_all = 4'b0001;

else if (!empty1)

grant_all = 4'b0010;

else if (!empty2)

grant_all = 4'b0100;

else if (!empty3)

grant_all = 4'b1000;

else

grant_all = 0;

end

S_FIFO1: begin

if (!empty1)

grant_all = 4'b0010;

else if (!empty2)

grant_all = 4'b0100;

else if (!empty3)

grant_all = 4'b1000;

else if (!empty0)

grant_all = 4'b0001;

else

grant_all = 0;

end

S_FIFO2: begin

if (!empty2)

grant_all = 4'b0100;

else if (!empty3)

grant_all = 4'b1000;

else if (!empty0)

grant_all = 4'b0001;

else if (!empty1)

grant_all = 4'b0010;

else

grant_all = 0;

end

S_FIFO3: begin

if (!empty3)

grant_all = 4'b1000;

else if (!empty0)

grant_all = 4'b0001;

else if (!empty1)

grant_all = 4'b0010;

else if (!empty2)

grant_all = 4'b0100;

else

grant_all = 0;

end

default: grant_all = 0;

endcase

end

endmodule

## 3.4 FIFO_Arbiter_System.v — FIFO + Arbiter Integration

module FIFO_Arbiter_System #(

parameter DATA_WIDTH = 40

) (

input clk, rst_n,

input wr_en0, wr_en1, wr_en2, wr_en3,

input [DATA_WIDTH-1:0] data_in0, data_in1, data_in2, data_in3,

input Router_Ready,

output empty0, empty1, empty2, empty3,

output full0, full1, full2, full3,

output grant0, grant1, grant2, grant3,

output reg [DATA_WIDTH-1:0] data_out,

output packet_valid

);

wire r_en0, r_en1, r_en2, r_en3;

assign r_en0 = grant0 && Router_Ready;

assign r_en1 = grant1 && Router_Ready;

assign r_en2 = grant2 && Router_Ready;

assign r_en3 = grant3 && Router_Ready;

wire [DATA_WIDTH-1:0] data_out0, data_out1, data_out2, data_out3;

Four_Fifo_Buffers #(

.DATA_WIDTH(DATA_WIDTH)

) FIFO_BUFFERS (

.clk(clk),

.rst_n(rst_n),

.wr_en0(wr_en0),

.wr_en1(wr_en1),

.wr_en2(wr_en2),

.wr_en3(wr_en3),

.r_en0(r_en0),

.r_en1(r_en1),

.r_en2(r_en2),

.r_en3(r_en3),

.data_in0(data_in0),

.data_in1(data_in1),

.data_in2(data_in2),

.data_in3(data_in3),

.empty0(empty0),

.empty1(empty1),

.empty2(empty2),

.empty3(empty3),

.full0(full0),

.full1(full1),

.full2(full2),

.full3(full3),

.data_out0(data_out0),

.data_out1(data_out1),

.data_out2(data_out2),

.data_out3(data_out3)

);

RR_Arbiter ARBITER (

.clk(clk),

.rst_n(rst_n),

.empty0(empty0),

.empty1(empty1),

.empty2(empty2),

.empty3(empty3),

.Router_Ready(Router_Ready),

.grant0(grant0),

.grant1(grant1),

.grant2(grant2),

.grant3(grant3)

);

reg [3:0] selected_fifo;

assign packet_valid = |selected_fifo;

always @(posedge clk) begin

if (!rst_n) begin

selected_fifo <= 4'b0000;

end

else if (Router_Ready) begin

selected_fifo <= {grant3, grant2, grant1, grant0};

end

end

always @(*) begin

case (selected_fifo)

4'b0001: data_out = data_out0;

4'b0010: data_out = data_out1;

4'b0100: data_out = data_out2;

4'b1000: data_out = data_out3;

default: data_out = {DATA_WIDTH{1'b0}};

endcase

end

endmodule

## 3.5 Packet_Routing.v — Packet Routing Module

module Packet_Routing #(

parameter DATA_WIDTH = 24

) (

input clk, rst_n,

input [DATA_WIDTH-1:0] packet_in,

input packet_valid,

input port0_ready, port1_ready, port2_ready, port3_ready,

output reg  [DATA_WIDTH-1:0] port0_data,  port1_data,  port2_data,  port3_data,

output reg port0_valid, port1_valid, port2_valid, port3_valid

);

localparam Dest0 = 2'b00,

Dest1 = 2'b01,

Dest2 = 2'b10,

Dest3 = 2'b11;

reg [DATA_WIDTH-1:0] packet_reg;

reg [1:0] destination_reg;

reg packet_active;

always @(posedge clk or negedge rst_n) begin

if (!rst_n) begin

packet_reg      <= {DATA_WIDTH{1'b0}};

destination_reg <= 2'b00;

packet_active   <= 1'b0;

end else begin

if (!packet_active) begin

if (packet_valid) begin

packet_reg      <= packet_in;

destination_reg <= packet_in[DATA_WIDTH-1 : DATA_WIDTH-2];

packet_active   <= 1'b1;

end

end else begin

case (destination_reg)

Dest0: if (port0_ready) packet_active <= 1'b0;

Dest1: if (port1_ready) packet_active <= 1'b0;

Dest2: if (port2_ready) packet_active <= 1'b0;

Dest3: if (port3_ready) packet_active <= 1'b0;

default: packet_active <= 1'b0;

endcase

end

end

end

always @(*) begin

port0_valid = 1'b0; port1_valid = 1'b0; port2_valid = 1'b0; port3_valid = 1'b0;

port0_data  = {DATA_WIDTH{1'b0}}; port1_data  = {DATA_WIDTH{1'b0}};

port2_data  = {DATA_WIDTH{1'b0}}; port3_data  = {DATA_WIDTH{1'b0}};

if (packet_active) begin

case (destination_reg)

Dest0: begin port0_valid = 1'b1; port0_data = packet_reg; end

Dest1: begin port1_valid = 1'b1; port1_data = packet_reg; end

Dest2: begin port2_valid = 1'b1; port2_data = packet_reg; end

Dest3: begin port3_valid = 1'b1; port3_data = packet_reg; end

endcase

end

end

endmodule

## 3.6 ram.v — Memory Node

// Memory node with destination parameter to route response packets to Port 3 (Testbench)

module ram_network_node_fixed #(

parameter DATA_WIDTH = 24,

parameter ADDR_WIDTH = 5,

parameter RAM_DEPTH  = 32,

parameter [1:0] RESP_DEST = 2'b11 // Destination for read response packets

) (

input clk, rst_n, rx_valid,

input [DATA_WIDTH-1:0] rx_data,

input fifo_full,

output reg rx_ready,

output reg  [DATA_WIDTH-1:0] tx_data,

output reg tx_wr_en

);

reg [15:0] mem [0:RAM_DEPTH-1];

localparam [1:0] S_IDLE      = 2'b00,

S_SEND_RESP = 2'b01,

S_DONE      = 2'b10;

reg [1:0] state;

reg [ADDR_WIDTH-1:0] saved_addr;

reg [15:0] saved_data;

wire       pkt_rw   = rx_data[21];

wire [ADDR_WIDTH-1:0] pkt_addr = rx_data[20:16];

wire [15:0] pkt_data = rx_data[15:0];

always @(posedge clk or negedge rst_n) begin

if (!rst_n) begin

state      <= S_IDLE;

rx_ready   <= 1'b1;

tx_wr_en   <= 1'b0;

tx_data    <= {DATA_WIDTH{1'b0}};

saved_addr <= {ADDR_WIDTH{1'b0}};

saved_data <= 16'h0000;

end else begin

case (state)

S_IDLE: begin

tx_wr_en <= 1'b0;

rx_ready <= 1'b1;

if (rx_valid && rx_ready) begin

if (pkt_rw == 1'b1) begin

mem[pkt_addr] <= pkt_data;

end else begin

saved_addr <= pkt_addr;

saved_data <= mem[pkt_addr];

rx_ready   <= 1'b0;

state      <= S_SEND_RESP;

end

end

end

S_SEND_RESP: begin

if (!fifo_full) begin

tx_data  <= {RESP_DEST, 1'b0, saved_addr, saved_data};

tx_wr_en <= 1'b1;

state    <= S_DONE;

end else begin

tx_wr_en <= 1'b0;

end

end

S_DONE: begin

tx_wr_en <= 1'b0;

rx_ready <= 1'b1;

state    <= S_IDLE;

end

default: state <= S_IDLE;

endcase

end

end

endmodule

## 3.7 network.v — Network System Top

// Structural wrapper wiring Ports 0, 1, 2 to memories and exposing Port 3 to TB

module Network_System_Top (

input clk, rst_n, Router_Ready, tb_wr_en,

input [23:0] tb_data_in,

input tb_port3_ready,

output wire [23:0] tb_port3_data,

output tb_port3_valid,

output tb_fifo3_full,

output tb_fifo3_empty

);

wire wr_en0, wr_en1, wr_en2;

wire [23:0] data_in0, data_in1, data_in2;

wire [23:0] port0_data, port1_data, port2_data;

wire port0_valid, port1_valid, port2_valid;

wire port0_ready, port1_ready, port2_ready;

wire full0, full1, full2;

wire empty0, empty1, empty2;

Router_Top #(.DATA_WIDTH(24)) router_inst (

.clk(clk),

.rst_n(rst_n),

.wr_en0(wr_en0), .wr_en1(wr_en1), .wr_en2(wr_en2), .wr_en3(tb_wr_en),

.data_in0(data_in0), .data_in1(data_in1), .data_in2(data_in2), .data_in3(tb_data_in),

.Router_Ready(Router_Ready),

.port0_ready(port0_ready), .port1_ready(port1_ready), .port2_ready(port2_ready), .port3_ready(tb_port3_ready),

.port0_data(port0_data), .port1_data(port1_data), .port2_data(port2_data), .port3_data(tb_port3_data),

.port0_valid(port0_valid), .port1_valid(port1_valid), .port2_valid(port2_valid), .port3_valid(tb_port3_valid),

.empty0(empty0), .empty1(empty1), .empty2(empty2), .empty3(tb_fifo3_empty),

.full0(full0),   .full1(full1),   .full2(full2),   .full3(tb_fifo3_full)

);

ram_network_node_fixed #(.DATA_WIDTH(24), .ADDR_WIDTH(5), .RAM_DEPTH(32), .RESP_DEST(2'b11)) node0 (

.clk(clk), .rst_n(rst_n),

.rx_valid(port0_valid), .rx_data(port0_data), .rx_ready(port0_ready),

.tx_wr_en(wr_en0), .tx_data(data_in0), .fifo_full(full0)

);

ram_network_node_fixed #(.DATA_WIDTH(24), .ADDR_WIDTH(5), .RAM_DEPTH(32), .RESP_DEST(2'b11)) node1 (

.clk(clk), .rst_n(rst_n),

.rx_valid(port1_valid), .rx_data(port1_data), .rx_ready(port1_ready),

.tx_wr_en(wr_en1), .tx_data(data_in1), .fifo_full(full1)

);

ram_network_node_fixed #(.DATA_WIDTH(24), .ADDR_WIDTH(5), .RAM_DEPTH(32), .RESP_DEST(2'b11)) node2 (

.clk(clk), .rst_n(rst_n),

.rx_valid(port2_valid), .rx_data(port2_data), .rx_ready(port2_ready),

.tx_wr_en(wr_en2), .tx_data(data_in2), .fifo_full(full2)

);

endmodule

## 3.8 Router_TopModule.v — Router Top Module

module Router_Top #(

parameter DATA_WIDTH = 24

) (

input clk, rst_n,

input wr_en0, wr_en1, wr_en2, wr_en3,

input [DATA_WIDTH-1:0] data_in0, data_in1, data_in2, data_in3,

input Router_Ready,

input port0_ready, port1_ready, port2_ready, port3_ready,

output [DATA_WIDTH-1:0] port0_data,  port1_data,  port2_data,  port3_data,

output port0_valid, port1_valid, port2_valid, port3_valid,

output empty0, empty1, empty2, empty3,

output full0,  full1,  full2,  full3

);

wire grant0, grant1, grant2, grant3;

wire [DATA_WIDTH-1:0] data_out;

wire packet_valid;

FIFO_Arbiter_System #(.DATA_WIDTH(DATA_WIDTH)) inst_arb (

.clk(clk),

.rst_n(rst_n),

.wr_en0(wr_en0),

.wr_en1(wr_en1),

.wr_en2(wr_en2),

.wr_en3(wr_en3),

.data_in0(data_in0),

.data_in1(data_in1),

.data_in2(data_in2),

.data_in3(data_in3),

.Router_Ready(Router_Ready),

.empty0(empty0),

.empty1(empty1),

.empty2(empty2),

.empty3(empty3),

.full0(full0),

.full1(full1),

.full2(full2),

.full3(full3),

.grant0(grant0),

.grant1(grant1),

.grant2(grant2),

.grant3(grant3),

.data_out(data_out),

.packet_valid(packet_valid)

);

Packet_Routing #(.DATA_WIDTH(DATA_WIDTH)) inst_rout (

.clk(clk),

.rst_n(rst_n),

.packet_in(data_out),

.packet_valid(packet_valid),

.port0_ready(port0_ready),

.port1_ready(port1_ready),

.port2_ready(port2_ready),

.port3_ready(port3_ready),

.port0_valid(port0_valid),

.port1_valid(port1_valid),

.port2_valid(port2_valid),

.port3_valid(port3_valid),

.port0_data(port0_data),

.port1_data(port1_data),

.port2_data(port2_data),

.port3_data(port3_data)

);

endmodule

# 4. Testbench

## 4.1 n_tb.v — Network Testbench

`timescale 1ns / 1ps

module tb_Network_System;

reg clk;

reg rst_n;

reg Router_Ready;

reg tb_wr_en;

reg [23:0] tb_data_in;

reg tb_port3_ready;

wire [23:0] tb_port3_data;

wire tb_port3_valid;

wire tb_fifo3_full;

wire tb_fifo3_empty;

Network_System_Top uut (

.clk(clk),

.rst_n(rst_n),

.Router_Ready(Router_Ready),

.tb_wr_en(tb_wr_en),

.tb_data_in(tb_data_in),

.tb_port3_ready(tb_port3_ready),

.tb_port3_data(tb_port3_data),

.tb_port3_valid(tb_port3_valid),

.tb_fifo3_full(tb_fifo3_full),

.tb_fifo3_empty(tb_fifo3_empty)

);

// 10ns Clock generation (100MHz)

always #5 clk = ~clk;

// Task to dispatch a write packet into Port 3 FIFO

task send_write(input [1:0] dest, input [4:0] addr, input [15:0] data);

begin

@(posedge clk);

while (tb_fifo3_full) @(posedge clk);

tb_data_in <= {dest, 1'b1, addr, data};

tb_wr_en   <= 1'b1;

@(posedge clk);

tb_wr_en   <= 1'b0;

end

endtask

// Task to dispatch a read request packet into Port 3 FIFO

task send_read(input [1:0] dest, input [4:0] addr);

begin

@(posedge clk);

while (tb_fifo3_full) @(posedge clk);

tb_data_in <= {dest, 1'b0, addr, 16'h0000};

tb_wr_en   <= 1'b1;

@(posedge clk);

tb_wr_en   <= 1'b0;

end

endtask

// Task to receive and check response on Port 3

task wait_read_response(input [4:0] exp_addr, input [15:0] exp_data);

begin

while (!tb_port3_valid) @(posedge clk);

if (tb_port3_data[20:16] === exp_addr && tb_port3_data[15:0] === exp_data) begin

$display("[PASS] TB Read Verification: Addr = 0x%0h | Data = 0x%04h",

tb_port3_data[20:16], tb_port3_data[15:0]);

end else begin

$display("[FAIL] TB Read Verification: Expected Addr = 0x%0h, Data = 0x%04h | Got Addr = 0x%0h, Data = 0x%04h",

exp_addr, exp_data, tb_port3_data[20:16], tb_port3_data[15:0]);

end

@(posedge clk);

end

endtask

initial begin

clk = 0;

rst_n = 0;

Router_Ready = 1;

tb_wr_en = 0;

tb_data_in = 24'd0;

tb_port3_ready = 1;

// Reset Sequence

#20;

rst_n = 1;

#20;

$display("--- Step 1: Writing data to memories 0, 1, and 2 ---");

send_write(2'b00, 5'd4,  16'hA1B2); // Node 0, Addr 4

send_write(2'b01, 5'd10, 16'hC3D4); // Node 1, Addr 10

send_write(2'b10, 5'd31, 16'hE5F6); // Node 2, Addr 31

#150;

$display("--- Step 2: Reading back data and checking responses on Port 3 ---");

send_read(2'b00, 5'd4);

wait_read_response(5'd4, 16'hA1B2);

send_read(2'b01, 5'd10);

wait_read_response(5'd10, 16'hC3D4);

send_read(2'b10, 5'd31);

wait_read_response(5'd31, 16'hE5F6);

#100;

$display("--- All Operations Finished ---");

$stop;

end

endmodule

# 5. Simulation Results

## 5.1 Waveforms

# 6. Synthesis

## 6.1 RTL Schematic

## 6.2 Resource Utilization